# SHORT GUIDE TO TROUBLESHOOTING

I. Messages on the screen like:

| Message on Screen | What may need to be done |
| --- | --- |
| A short has been detected in your card. | Replace IRD |
| Access card has expired. | Replace card |
| Call extension 711 | Try hard boot (unplug for 10 Min) We are assuming that the signal is above 75 and Programming is in the card…Call ISS if not successful |
| Call extension 721 | Call for authorization |
| Call extension 722 | Try hard boot…Call ISS |
| Call extension 731 | Do a phone test (ok)/ call for a forced download |
| Call extension 732 | Do a phone test (ok)/ call for a forced download/ check for YES on PPV |
| Call extension 733 | Replace or repair the IRD |
| Call extension 734 | Call for rehit/replace the card…Call ISS |
| Call extension 741-742 | Call to add PPV |
| Call extension 745 | Hard boot (signal above 75/ programming is in the card)…Call ISS |
| Call extension 746 | Hard boot (signal above 75/ programming is in the card)…Call ISS |
| Channel not available | If channels are in the line-up, do a hard boot |
| Information not available | Check the dish pointing and select dish to Oval satellite dish with 2 sat. locations |
| Please insert a valid access card | If card is not damaged, do a hard boot/ replace card |
| Searching for satellite signal | Check LNB, IRD, fittings and cables |
| You have inserted the wrong card | Call ISS |

II. Other issues on the installation scene:

Missing transponders			Check fittings/ cables/ LNB

To find the video input on the TV	On RCA, GE, or PROSCAN press TV on the remote control that
 came with your DirecTV Plus System. Press the WHO.INPUT
 button on the remote some channels for video inputs are:
 00-90-91- 92- VID 1- VID 2

Satellite 101				Transponders 1to 32

Satellite 110				Transponders 8, 10 and 12

Satellite 119				Transponders 22 to 32

DirecTV plus and Paratodos system		Verify channel 100 for all satellites/ ch.90 for 110/
                                                                         ch.400 for 119 sat.

III. Phone test / Signal Test:

BELL ATLANTIC				Main menu / Setup menu / Installation menu / Sys.test

HNS	HIRD-A2; HIRD-A3			Action/ Guide/ Main menu/ Setup menu/ Sys.test

HIRD-A4				Action/ Guide/ Main menu/ Setup menu/ Installation 
                                                          menu/ Sys.test

HIRD-B1; B2; B3; B4; C2			Action/ Guide/ Main menu/ Setup/ Installation
                                                                                       menu/ Sys.test

HIRD-D0; D1; D2; D4; D5		Action/ Guide/ Main menu/ Setup menu/
                                                                                       Installation/ Sys.test
HIRD-E1; E2; E6  (multi-Sat)		Action/ Guide/ Main menu/ Setup menu/ 
                                                                                      Installation/ System test

RCA	DRD001RW				Main menu/ Options/ Setup/ Sys.test/ Test

DRD112NW

DRD212NW

DRD213NW

DRD102RW

DRD203RW

NRD313NA				Main menu/ Options/ Sys.test

NRD412NA

DRD202RA; DCD302RA			Main menu/ Options/ Sys.test

DRD302RA; DRD303RA

DRD403RA; DRD703RA

DC403RB; DRD502RB			Main menu/ Options/ System options/ Sys.test

DRD503RB; DRD503RBC

DRD505RB; DRD515RB

DRD523RB; DRD523MCDC

DRD705RB; NRD513NB

DRD221RD; DRD222RD

DRD223RD; DRD225RD

NRD205RD; DRD420RE

DRD440RE; DRD460RE

DRD480RE

SONY	SAT A-1; SAT B-1			Main menu/ Sys.menu/ Installation menu/
                                                                                       Run sys.test/ Test

SAT A-2; SAT B-2			Main menu/ Setup menu/ Sys.test/ Test

SAT A-3; SAT B-3			Main menu/ Setup menu/ Sys.test/ Test

SAT A-4				Main menu/ Setup menu/ Sys.test/ Test

SAT A-50; SAT B-50			Main menu/ System, option # 1/ Sys.test/ Test

SAT A-55; SAT B-55			Main menu/ System/ Sys.test/ Test